// --- Libraries ---

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <EEPROM.h>  // Optional: for calibration storage if needed

// --- LCD Setup ---

LiquidCrystal_I2C lcd(0x27, 16, 2);  // I2C address 0x27, 16 columns, 2 rows

// --- Pin Definitions ---

#define TDS_SENSOR_PIN 34      // Analog input for TDS sensor

#define FLOAT_SENSOR_PIN 15    // Digital input for float sensor

// --- Constants ---

#define VREF 3.3                // ESP32 analog reference voltage

float calibrationFactor = 1.0; // Adjust based on your TDS sensor calibration

float temperature = 25;        // Default temp for compensation

// --- Variables ---

float tdsValue = 0;

void setup() {

// Initialize serial communication

Serial.begin(115200);

// Setup pin modes

pinMode(TDS_SENSOR_PIN, INPUT);

pinMode(FLOAT_SENSOR_PIN, INPUT_PULLUP);  // Float sensor uses pull-up

// Initialize LCD

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("System Initializing");

delay(2000);

lcd.clear();

Serial.println("System Initialized: TDS + Float Sensor + LCD");

}

void loop() {

// --- TDS Sensor Reading ---

int analogValue = analogRead(TDS_SENSOR_PIN);

float voltage = analogValue * (VREF / 4095.0);

tdsValue = (133.42 * voltage * voltage * voltage

- 255.86 * voltage * voltage

+ 857.39 * voltage) * calibrationFactor;

// --- Float Sensor Reading ---

int floatSensorState = digitalRead(FLOAT_SENSOR_PIN);

String floatStatus = (floatSensorState == LOW) ? "Water: FULL" : "Water: LOW";

// --- Serial Output ---

Serial.println("----- Sensor Readings -----");

Serial.print("TDS Analog Value: ");

Serial.print(analogValue);

Serial.print(" | Voltage: ");

Serial.print(voltage, 2);

Serial.print("V | TDS: ");

Serial.print(tdsValue, 0);

Serial.println(" ppm");

Serial.println("Float Sensor: " + floatStatus);

Serial.println("---------------------------\n");

// --- LCD Output ---

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("TDS: ");

lcd.print((int)tdsValue);

lcd.print(" ppm");

lcd.setCursor(0, 1);

lcd.print(floatStatus);

delay(2000);  // Wait 2 seconds between readings

}